Industrial Machine Maintenance

& Spare-Parts Analysis Report

Prepared for: Plant Operations & Maintenance Leadership

Data period: 3 January 2022 – 1 January 2025 | 10 Assets | 3 Plants

Prepared by: Data Analytics Team | 25 July 2026

# 1. Executive Summary

This report analyses three years of daily sensor and spare-parts transaction data across 10 machines (5 machine types) in 3 plants, to understand where breakdowns happen, what they cost, and what warning signs appear before they occur.

## Key numbers at a glance

| Metric | Value |
| --- | --- |
| Total spare-parts spend | ₹1,33,40,945 (₹1.33 Crore) |
| Total work orders raised | 42,793 |
| Breakdown (unplanned) work orders | 21,636 (50.6%) |
| Planned maintenance work orders | 21,157 (49.4%) |
| Machines tracked | 10, across CNC Lathe, Hydraulic Press, Belt Conveyor, Screw Compressor, EOT Crane |
| Plants covered | PUN-01 (Pune), CHN-02 (Chennai), DHR-03 (Dhar) |
| Highest-spend part family | Bearings (₹32.60 Lakh, 24.4% of spend) |
| Costliest single part | Hydraulic Seal Kit 50mm Bore (₹28.48 Lakh) |

## Top takeaways

- Reactive (breakdown) maintenance is almost as common as planned maintenance — nearly 1 in 2 work orders is a breakdown, which is a strong signal that preventive schedules are not catching problems early enough.

- Hydraulic Press and Screw Compressor machine types have the highest breakdown work-order counts, and are also close to the top in spend — they deserve first priority for a condition-monitoring push.

- Bearings and Seals & Gaskets together account for nearly half of total spare-parts spend and also lead in breakdown counts, making them the biggest cost and reliability lever.

- On breakdown days, machines run noticeably hotter, more loaded and draw more power — power consumption is 12.5% higher and load is 6.3% higher than on normal days, making these two the most useful early-warning signals.

- Plant DHR-03 has the highest breakdown rate (69.0%) among the three plants, even though it has fewer assets than PUN-01 — worth a local review of maintenance practices.

- Spend is fairly stable month-to-month (₹3–4.8 Lakh/month) but breakdown counts show mild seasonal bumps, generally rising toward Dec–Jan each year.

# 2. About the Data

The dataset contains 2,19,000 daily records covering 10 machines, each tracked with 20 spare-part lines, sensor readings and work-order details over a 3-year window.

## What's in the data

| Field group | Examples |
| --- | --- |
| Machine identity | Asset tag, machine type, plant code, criticality (A/B/C) |
| Parts & cost | Part number, description, family, unit cost, quantity issued, issue value |
| Condition sensors | Bearing temp, motor temp, horizontal & vertical vibration, oil pressure, load %, shaft RPM, power (kW) |
| Maintenance events | Breakdown flag, work order type (Breakdown / Planned) |

## Machine fleet covered

- 5 machine types, 2 machines each: CNC Lathe, Hydraulic Press, Belt Conveyor, Screw Compressor, EOT Crane

- 3 plants: PUN-01 (4 machines), CHN-02 (3 machines), DHR-03 (3 machines)

- 9 spare-part families: Bearing, Seal & Gasket, Drive Belt, Filter, Lubrication, Sensor, Fastener, Electrical, Coupling

# 3. Spare-Parts Spend Analysis

Total spend across the 3-year period is ₹1.33 Crore. This section breaks it down by part family, plant, machine type and the top individual parts driving cost.

Figure 1: Where the spend goes — Bearings and Seal & Gasket parts together make up ~47% of all spend.

## Spend by plant

Figure 2: PUN-01 has the highest total spend, but it also runs the most machines (4 of 10).

## Spend by machine type and criticality

| Machine Type | Spend (₹) | % of Total |
| --- | --- | --- |
| Belt Conveyor | 27,45,522 | 20.6% |
| Screw Compressor | 27,14,335 | 20.3% |
| EOT Crane | 26,42,723 | 19.8% |
| Hydraulic Press | 26,32,803 | 19.7% |
| CNC Lathe | 26,05,562 | 19.5% |

- Criticality-A (most critical) assets account for ₹68.87 Lakh — 51.6% of total spend — despite being a smaller share of the fleet, confirming spend is correctly weighted toward critical equipment.

- Criticality-B assets: ₹49.34 Lakh (37.0%); Criticality-C: ₹15.20 Lakh (11.4%).

## Top 10 parts by spend

Figure 3: The Hydraulic Seal Kit alone accounts for over ₹28 Lakh — more than double the next item.

# 4. Breakdown & Work Order Analysis

Figure 4: Roughly half of all work orders are unplanned breakdowns rather than planned maintenance — a sign of a largely reactive maintenance approach.

## Breakdowns by machine type

Figure 5: Hydraulic Press and Screw Compressor lead in breakdown work orders.

## Breakdowns by asset criticality

Figure 6: Criticality-B assets actually see the most breakdown events (43.3%), ahead of Criticality-A (38.8%) — worth investigating why 'less critical' assets are breaking down more.

## Breakdowns by part family

| Part Family | Breakdown Work Orders | Share |
| --- | --- | --- |
| Bearing | 4,221 | 19.5% |
| Seal & Gasket | 3,118 | 14.4% |
| Sensor | 2,454 | 11.3% |
| Electrical | 2,417 | 11.2% |
| Filter | 2,136 | 9.9% |
| Drive Belt | 2,134 | 9.9% |
| Coupling | 1,987 | 9.2% |
| Fastener | 1,626 | 7.5% |
| Lubrication | 1,543 | 7.1% |

Bearings are the single biggest contributor to breakdowns, consistent with them also being the top spend item — reinforcing bearings as the number-one focus area for reliability improvement.

## Which specific assets break down most

| Asset Tag | Breakdown Work Orders |
| --- | --- |
| AST-2017 | 2,647 |
| AST-5003 | 2,565 |
| AST-4055 | 2,558 |
| AST-2031 | 2,311 |
| AST-4061 | 2,102 |

These five assets alone account for over half of all breakdown events and are strong candidates for a focused root-cause review or replacement study.

# 5. Early-Warning Signals: Sensor Behaviour Before Breakdown

Comparing average sensor readings on days when a breakdown occurred against normal days shows which signals are the most reliable early warning indicators.

Figure 7: Power consumption and load % rise the most on breakdown days, making them the strongest predictive signals; temperature and vibration shift only slightly.

| Sensor | Normal Days (avg) | Breakdown Days (avg) | % Higher |
| --- | --- | --- | --- |
| Power consumption (kW) | 26.13 | 29.40 | +12.5% |
| Load (%) | 67.92 | 72.18 | +6.3% |
| Vertical vibration (mm/s) | 2.89 | 3.02 | +4.5% |
| Horizontal vibration (mm/s) | 3.85 | 4.02 | +4.4% |
| Oil pressure (bar) | 5.00 | 5.08 | +1.6% |
| Motor temperature (°C) | 71.06 | 71.33 | +0.4% |
| Bearing temperature (°C) | 65.08 | 65.25 | +0.3% |

## What this means

- Power consumption is the single most useful leading indicator in this dataset — a sustained rise is worth flagging for inspection before a breakdown work order is raised.

- Load % moving up in tandem with power draw strengthens the signal — the two together could form a simple two-variable alert rule.

- Temperature and vibration shift only marginally around breakdown events in this data, so on their own they are weaker predictors here, though they remain standard safety and condition checks.

# 6. Trend Over Time

Figure 8: Monthly spend (blue, left axis) versus breakdown work orders (red, right axis), Jan 2022 – Dec 2024.

Spend stays in a fairly narrow band of ₹3.0–4.8 Lakh per month, with recurring spikes around March, June, September and December — consistent with quarterly planned-maintenance cycles. Breakdown work orders fluctuate between roughly 500 and 690 per month without a strong long-term upward or downward trend, meaning the underlying breakdown rate has stayed broadly steady across the 3 years rather than improving.

# 7. Recommendations

## Immediate actions

- Prioritise a reliability review on the top 5 breakdown-prone assets (AST-2017, AST-5003, AST-4055, AST-2031, AST-4061) — they drive over half of all breakdown events.

- Set up a monitoring alert on power consumption and load % — these showed the clearest gap between normal and breakdown days and are the most actionable early-warning pair.

- Standardise bearing replacement intervals — bearings are simultaneously the top spend item and the top breakdown contributor, so small reliability gains here have outsized cost impact.

## Medium-term actions

- Shift the maintenance mix from reactive to planned — with breakdown work orders at 50.6% of the total, moving even 10-15 percentage points toward planned maintenance should reduce both downtime and emergency-purchase costs.

- Review maintenance practices at plant DHR-03, which has the highest breakdown rate (69.0%) despite a smaller fleet than PUN-01.

- Investigate why Criticality-B assets show more breakdowns than Criticality-A assets — criticality classification may need to be revisited, or B-class assets may be under-maintained relative to their actual duty.

## Data & process suggestions

- Extend sensor logging to include a derived 'load-power ratio' metric, which may sharpen the early-warning signal further.

- Track mean-time-between-failures (MTBF) per asset going forward to complement this spend/breakdown-count view with a reliability-engineering perspective.

— End of Report —